Land Surface Temperature (LST) Calculation & Statistics
Shimla District (2015–2025)

# 1. Formulas Used

For each year (2015 and 2025):

Spectral radiance = ML × Q cal + AL

Brightness Temperature (Kelvin) = BT = K2 / ln((K1 / Lλ) + 1)

BT (Celsius) = BT − 273.15

NDVI = (B5 − B4) / (B5 + B4)

Proportion of vegetation =  PV = ((NDVI − NDVImin) / (NDVImax − NDVImin))²

Emissivity:  ε = 0.004 × PV + 0.986

LST (Celsius) =  LST = BT / (1 + (λ × BT / ρ) × ln(ε))

# 2. Constants (from MTL.txt)

Scene-specific constants from MTL.txt:

For (2015):

| Constant name | Symbol | Value (from MTL) |
| --- | --- | --- |
| RADIANCE_MULT_BAND_10 | ML | 0.000342 |
| RADIANCE_ADD_BAND_10 | AL | 0.10000 |
| K1_CONSTANT_BAND_10 | K1 | 774.8853 |
| K2_CONSTANT_BAND_10 For (2025): | K2 | 1321.0789 |
| Constant name | Symbol | Value (from MTL) |
| RADIANCE_MULT_BAND_10 | ML | 0.00038 |
| RADIANCE_ADD_BAND_10 | AL | 0.10000 |
| K1_CONSTANT_BAND_10 | K1 | 799.0284 |
| K2_CONSTANT_BAND_10 | K2 | 1329.2405 |

Commonly used constants for the emissivity correction term:
• λ (effective wavelength for Band 10) ≈ 10.895 µm
• ρ ≈ 1.438 × 10⁻² m· K (sometimes written as 1.4388×10⁻²)

3. Map Ranges Used (for visual comparability)

To compare 2015 vs 2025 visually, a common legend range was used.

• LST map legend range (both years): 20 to 33 °C

• LST change map legend range (2025 − 2015): -2 to 4 °C


4. Conclusion – Land Surface Temperature Analysis

The Land Surface Temperature (LST) maps derived from Landsat 8 data show clear spatial variation across Shimla district. In 2015, surface temperatures ranged approximately between 20 °C and 28 °C, while in 2025 the temperatures extended up to about 33 °C. For consistent visual comparison, a common legend range of 20 °C to 33 °C was applied to both maps.

The comparison indicates a shift toward higher surface temperatures in several areas, particularly in lower elevation regions and locations with relatively sparse vegetation. In contrast, forested and higher altitude areas show relatively lower temperatures. Overall, the results indicate a moderate increase in surface temperature between 2015 and 2025, which aligns with the spatial patterns observed in the NDVI and vegetation change maps.